Project 1 Report

I. Introduction

UNMC has comprised a working group of health professionals, environmental scientists, and engineers to work on how pollution can negatively impact health. UNMC has partnered with AirPurple to help monitor pollution across Nebraska. Air quality is not a specialty of anyone currently affiliated with UNMC, so they have opened a request for proposals from engineering firms to help UNMC analyze the air quality data set. Our engineering firm won the proposal and is now tasked with completing the data analysis.

The goal of this project is to provide an analysis of air quality data from across Nebraska for a request for proposals from the UNMC Water, Climate, and Health working group. The client has asked for the following analyses: (1) an overall statistical analysis to determine the five locations with the highest mean and median concentrations of VOC, PM 2.5, and PM 10.0, (2) an analysis of when and where maximum values of VOC, PM 2.5, and PM 10.0 occurred, and research of potential causes, (3) an analysis into the effects temperature and humidity has on air quality, and (4) find instances of Air Quality Index (AQI) health risks at any location for PM 2.5 and PM 10.0 based on the EPA’s AQI ratings.

II. Methods

This data analysis was conducted using python to organize data and calculate statistics. The first task was a statistical analysis to determine the five sites with the greatest mean and median concentration of the three particulates. First, the data was imported as a .csv file and transformed into a data frame using the Pandas python package. The data was then grouped with the .groupby() function based on location. Using the .agg(), .sort_values(), and .head() functions, the top five locations of PM2.5, PM10.0 and VOCs was found. The .agg() function was used to find the mean and median values of particulates at each location. The .sort_values() function was used to order the data from highest number of particulates to lowest. The .head() function was used to take the top five results. Six total tables were created. Two for each of the three particulates, one to find the mean, and one to find the median.

The second task was to find the maximum values of each of the three particulates. As well as the date and location it occurred. First, the data imported in task one was filtered so that only the relevant columns, ‘date’, ‘sensor.name’, and the particulate we want to look at, are present. This was done using Pandas column selection. The data was then sorted using .sort_values() by highest number of particulates. Finally, the .head() function was used to take the top result. Three tables were created, one for each particulate.

The third task was to determine the effect temperature and humidity had on air quality. First, two functions were defined. Using if-statements, they took the temperatures and humidities from the data and grouped them into ranges. (For temperature: <32F, 32-50F, 50-70F, >70F. For humidity: <50%, 50-80%, >80%). After the functions were defined, they were applied to the data to add two new columns for the temperature and humidity group. Two data groups were then created using the .groupby() function. One grouped by the temperature level, and one for the humidity level. Finally, the .agg() function was used to find the mean and median of each particulate for each temperature and humidity group. Six total tables were created. One for each of the three particulates for temperature and one for each of the three particulates for humidity.

Task 4 was to find instances of AQI health risks of PM 2.5 and PM 10.0. Using the .loc[] function, the data was reduced to ‘date’, ‘sensor.name’, and particulate columns, as well as filtered  by the EPA’s guidelines (34.4 PM 2.5, 155 PM 10.0). The data was then sorted by highest number of particulates using .sort_values(). Two tables were created. One for health risks of PM 2.5, and one for health risks of PM 10.0.

III. Results

Task 1: In task one, the five locations with the highest mean and median amounts of  VOCs, PM 2.5, and PM 10.0 were found. The following are the results:

Mean VOCs: 1. Swnphd-ogallala, 2. FCHD-YPS, 3. Three Rivers Public Health Department, 4. ELVPHD Norfolk HD 4, 5. Swnphd-mccook.

Median VOCs: 1. Swnphd-ogallala, 2. Swnphd-mccook, 3. Three Rivers Public Health Department, 4. FCHD-YPS, 5. ELVPHD Norfolk HD 4.

Mean PM 2.5: 1. Broken Bow, 2. #16 - Richardson County Courthouse, 3. #18 - Southeast District Health Department- Tecumseh, 4. NCDHD O'Neill #11, 5. Swnphd-mccook.

Median PM 2.5: 1. Broken Bow, 2. #16 - Richardson County Courthouse, 3. #18 - Southeast District Health Department- Tecumseh, 4. ELVPHD Norfolk HD 4, 5. ELVPHD Wisner HD 5.

Mean PM 10.0: 1. Broken Bow, 2. #16 - Richardson County Courthouse, 3. #18 - Southeast District Health Department- Tecumseh, 4. NCDHD O'Neill #11, 5. Swnphd-mccook.

Median PM 10.0: 1. Broken Bow, 2. #16 - Richardson County Courthouse, 3. ELVPHD Norfolk HD 4, 4. #18 - Southeast District Health Department- Tecumseh, 5. ELVPHD Wisner HD 5.

Task 2: In task two, the maximum values of VOCs, PM 2.5, and PM 10.0 and possible causes were found. The following are the results:

VOCs: Swnphd-ogallala on 6/24/24.

This maximum was likely due to a severe grassland fire in western Nebraska near Oglala National Grasslands (Nebraska Public Media, 2024).

PM 2.5: #16 - Richardson County Courthouse on 2/28/25.

PM 10.0: #16 - Richardson County Courthouse on 2/28/25.

Both the PM 2.5 and PM 10.0 maximums happened on the same day in the same place. It’s unclear what could have caused this high reading. There weren’t any AQI advisories for the area around the date specified. The most likely explanation is that the data is flawed because of some sensor errors. That February was exceptionally cold with temperatures dropping below 0 multiple times (Luke, 2025). It’s likely that the cold damaged the sensors. After further investigation into the data, after 12/10/24, the PM 2.5 didn’t drop below 1500 all the way until the end of the data on 3/5/25. This leads me to believe that there was something massively wrong with the sensor and the data.

Task 3: In task 3, the effects of temperature and humidity were found. The following are the results:

Temperature in VOCs:

Mean		Median

Freezing	257.628626	251.385875

Cool		288.558923	283.105000

Warm		255.179179	225.691667

Hot		282.145818	237.298958

Temperature in PM 2.5:

Mean		Median

Freezing	273.698560	8.957875

Cool		143.167452	6.138187

Warm		85.914999	3.489583

Hot		70.145543	7.924854

Temperature in PM 10.0:

Mean		Median

Freezing	276.080794	11.236998

Cool		144.593363	7.517250

Warm		86.834046	4.349479

Hot		71.821037	9.223000

Looking at the statistical analysis for the temperature groups, a couple things stand out. First, the averages for PM 2.5 and PM 10.0 are massively skewed. That probably has something to do with the errors suspected in the last task. Back to the specifics of this task, it looks like temperature doesn’t affect VOCs, but it does effect the PM 2.5 and PM 10.0. It appears that both  mean PM 2.5 and PM 10.0 are inversely proportional to the temperature. However, the median of those do not seem to show that same relationship. The median of the data shows there are more PM 2.5 and PM 10.0 at extreme temperatures (high or low).

Humidity in VOCs:

Mean		Median

High		19.395555	0.000000

Medium	252.664069	197.070417

Low		279.329347	258.075292

Humidity in PM 2.5:

Mean		Median

High		533.786399	8.931062

Medium	80.874444	9.870667

Low		76.513935	5.638146

Humidity in PM 10.0:

Mean		Median

High		536.520850	11.872094

Medium	82.561133	11.062667

Low		77.880520	6.676271

Looking at the analysis for humidity, a few things stand out. First, for VOCs there seems to be a correlation. There are more VOCs at a lower humidity. It appears to be the opposite for PM 2.5 and PM 10.0. At higher humidities there are more PM 2.5 and PM 10.0.

Task 4: In task 4, AQI health risks were found. There were 574 instances of PM 2.5 AQI risks, and 391 instances of PM 10.0 risks. Overall, these health risks were most likely caused by fires both in state, and out of state. Prescribed agricultural burns in Nebraska and other states can cause AQI health risks. This is in addition to wildfires. These factors can all cause AQI risks.

IV. References

Wildfire burning in Western Nebraska. (2024). Nebraska Public Media. https://nebraskapublicmedia.org/en/news/news-articles/wildfire-burning-in-western-nebraska/

Luke, R., & Hunt, E. (2025, April 18). February 2025 Climate Summary [Review of February 2025 Climate Summary]. University of Nebraska - Lincoln; Nebraska State Climate Office. February 2025 Climate Summary | Nebraska State Climate Office | Nebraska. (2025). Unl.edu. https://nsco.unl.edu/news/february-2025-climate-summary/